JON_FIRST_REAL_PHYSICAL_TEST_PROTOTYPE_MASTER

FINAL P0 PRE-FABRICATION DESIGN + DOE / TEST READINESS MASTER

P0 = FIRST REAL PHYSICAL TEST PROTOTYPE

BUILD → TEST → MEASURE → LEARN → MODIFY → RETEST

R4.1 = IMMUTABLE

R4.1 SHA256 = fbe6b17cdbf7282a2e47963e567e12eeceb1352a36e719e7d1c55cc5f712a0a68

# 0. FINAL GATE POSITION

| Gate | Status | Meaning |
| --- | --- | --- |
| GATE 1 — P0 EXPLORATORY BUILD | CONDITIONAL / BLOCKED FOR EXACT RELEASE | Exploratory article is permitted in principle; exact fabrication cannot be released without native-CAD control of critical geometry and interfaces. |
| GATE 2 — P0 PHYSICAL TEST | NOT STARTED | No physical hardware is yet available; data validity cannot be claimed. |
| GATE 3 — DESIGN VALIDATION | NOT CLAIMED | P0 will not by itself establish vehicle/occupant validation. |
| P0 DOE DISCRIMINATION | PASS | H1–H7 each have a defined physical question, measurable outputs, and a decision consequence; P0-E is explicitly non-causal. |

Ruthless finding: the experiment design is no longer the weak link. The remaining blocker is source-controlled definition of the actual physical article: critical geometry, interfaces, fixture attachment, and released measurement hardware configuration. Inventing any of these would corrupt the test program.

# A. P0 ARCHITECTURE

P0 is an exploratory physical mechanism article, not a production seat and not an exact R4.1 reproduction. The architecture is retained unless a critical defect is physically or geometrically proven.

| Module | P0 role | Experimental status |
| --- | --- | --- |
| Primary base / fixture interface | Carries loads into the controlled test fixture | Real structural interface; exact dimensions CAD-dependent |
| Translation module S1 / S2 | Provides the controlled translation path and measurable reactions | Real mechanism; kinematic dimensions CAD-dependent |
| Seatback rotation-control module | Controls/records seatback rotation | Real mechanism; geometry must come from source definition |
| Absorber module | Exchangeable energy-management element | Modular / replaceable |
| Lock module | Engage / release / transition behavior | Modular / replaceable where mechanically appropriate |
| Rebound-control module | Controls return/rebound motion | Adjustable or replaceable |
| Joint / compliance interfaces | Allows controlled compliance and deformation measurement | Replaceable / adjustable where test-relevant |
| Stops / stroke interfaces | Defines controlled travel / end conditions | Replaceable where safe |
| Instrumentation interfaces | Provides repeatable sensor datums and brackets | P0 test hardware; not concept architecture |

# B. P0_HYPOTHESIS_REGISTER

| ID | Mechanism | Design variable | Controls | Measured outputs | Expected trend | Success criterion | Failure criterion | Unknown / test | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H1 | Controlled translation / ride-down | Translation-control mechanism | geometry, mass, BC, fixture, loading, initial condition | x(t), v(t), a(t), S1, S2 | Controlled, continuous motion rather than binding or unintended release | CHARACTERIZATION unless a defined requirement exists | Binding, discontinuity, uncontrolled travel, stroke-end behavior | L2→L4 | Retain / modify translation path only from evidence |
| H2 | Seatback rotation-control | Rotation-control mechanism | seat geometry, mass, BC, loading, fixture, initial state | θ(t), reaction forces, timing | Rotation follows intended controlled sequence | CHARACTERIZATION unless a defined requirement exists | Nonlinear region, loss of control, interference | L2→L4 | Retain / modify control geometry |
| H3 | Absorber behavior | Absorber variant | all non-absorber variables fixed | F_abs(t), stroke(t), F-x, F-v, system acceleration, reactions | Variant changes force/stroke/rate response measurably | CHARACTERIZATION unless threshold exists | rate dependence, hysteresis, bottom-out, instability | L3→L4 | Select variant / redesign interface |
| H4 | Lock transition | Lock variant / strategy | all non-lock variables fixed | lock state, transition timing, x/v/a, S1/S2 | Transition occurs at intended stage without unintended load-path change | CHARACTERIZATION unless lock requirement exists | premature transition, fail-to-lock, fail-to-unlock | L2→L4 | Retain / modify lock |
| H5 | Rebound control | Rebound-control element | geometry, mass, initial state, BC, fixture fixed | rebound excursion, v(t), a(t), timing | Variant changes rebound magnitude/timing in predicted direction | CHARACTERIZATION unless criterion exists | overshoot, delayed return, instability | L2→L4 | Retain / modify rebound control |
| H6 | Joint / compliance behavior | Joint/interface parameter | geometry, preload, hardware, BC fixed | joint deformation, relative motion, S1/S2 | Response is attributable to the single joint variable | CHARACTERIZATION | elastic/slip/yield transition, preload loss | L3→L4 | Modify joint/preload/interface if evidenced |
| H7 | Integrated mechanism interaction | Integrated P0-E state | system setup fixed; no causal claim | all synchronized channels | System behavior reveals interactions/non-additivity | SYSTEM CHARACTERIZATION | coupled effects, sequencing, emergent behavior | L5 | Generate next revision; never assign component causality from P0-E alone |

# C. P0 CONFIGURATION MATRIX

| Config | What changed? | What did not change? | Why / question | Discriminating data |
| --- | --- | --- | --- | --- |
| P0-A | Nothing experimental; baseline/control | All defined geometry, mass, fixture, BC, loading, instrumentation, initial state | Establish physical baseline behavior | Complete synchronized baseline signature |
| P0-B | Absorber A ↔ B only | Everything outside absorber variable | Does absorber variant materially change response? | Absorber F/stroke/F-x/F-v + global response |
| P0-C | Lock A ↔ B only | Everything outside lock variable | Does lock variant alter transition or load path? | Lock state/timing + motion + reactions |
| P0-D | Rebound-control A ↔ B only | Everything outside rebound variable | Does rebound-control alter excursion/timing/stability? | Rebound excursion + v/a + reactions |
| P0-E | Integrated system state | No component-isolation claim | How do mechanisms interact as a system? | Complete synchronized dataset; non-causal interpretation |

A/B discipline: ONE PRIMARY DESIGN VARIABLE. Any comparison that requires multiple simultaneous changes is explicitly tagged MULTIVARIABLE COMPARISON and cannot support causal attribution.

# D. DOE / TEST LOGIC

| Level | Primary test | Question answered | Minimum useful outputs | Decision impact |
| --- | --- | --- | --- | --- |
| L1 | Dimensional / assembly inspection | Did we build and assemble the intended P0 state? | critical dimensions, assembly state, preload/torque records, clearances | release to L2 or correct article |
| L2 | Functional / mechanical motion | Does the mechanism move, lock, rotate, rebound and reset as intended? | x/θ, lock state, contact observations, motion traces | stop / correct obvious physical defects / proceed |
| L3 | Quasi-static characterization | What are force/displacement/compliance characteristics? | force, displacement, deformation, absorber F-x | identify mechanism characteristics |
| L4 | Controlled dynamic bench | Can A/B variants be discriminated under controlled loading? | synchronized x/v/a, S1/S2, absorber, lock, rebound | retain/modify selected component |
| L5 | Integrated instrumented test | What interactions emerge when modules are combined? | all required synchronized channels | generate next design hypothesis / revision |
| L6 | Sled | Does evidence justify vehicle-like dynamic boundary testing? | defined sled-level measurements | conditional gate only |
| L7 | Higher energy | Is further escalation justified by prior evidence? | test-specific | separate technical/safety gate |

No level is skipped because an earlier result “looks promising.” The sequence is a release ladder, not a confidence ladder.

# E. P0_INSTRUMENTATION_PLAN

| Output | Sensor / method | Location / frame | Range / bandwidth / resolution | Sync / calibration / uncertainty | Raw + reduction |
| --- | --- | --- | --- | --- | --- |
| x(t) | Displacement/position measurement | Primary translation coordinate; fixture datum | Must resolve full motion without saturation; bandwidth sufficient for expected event | Calibrated; common timebase; uncertainty documented | Raw channel + validated position reduction |
| v(t) | Direct velocity or validated derivative of x(t) | Same motion coordinate | Derivative filtering must not erase physical events | Common timebase; uncertainty includes processing | Raw/derived + filter method |
| a(t) | Accelerometer | Defined structural/inertial point | Range and bandwidth must exceed expected response without clipping | Calibration + axis transform + synchronized timebase | Raw acceleration + validated filtering |
| seatback θ(t) | Encoder / validated angular measurement | Seatback reference frame | Resolution sufficient to distinguish transition/control events | Calibrated angular datum + common timebase | Raw angle + transformed θ(t) |
| S1 / S2 | Load measurement in rail/load path | Each defined reaction path | Range/resolution sufficient for peaks and low loads | Calibration trace + synchronized channels | Raw force + uncertainty/reduction |
| absorber force | Load cell / calibrated load path | Absorber force axis | Must capture low-force region and peak response | Calibration + force-axis definition | Raw force + F-x / F-v |
| absorber displacement | Stroke measurement | Absorber axis | Full valid stroke + adequate resolution | Calibrated; synchronized | Raw displacement + stroke result |
| joint deformation | LVDT/optical/strain-equivalent method | Joint/local datum | Must distinguish expected compliance from slip/yield | Calibrated; frame-defined | Raw + deformation reduction |
| lock state | Position/contact/state sensor | Lock mechanism | Unambiguous state discrimination; no missed transition | Calibrated; synchronized | State channel + transition time |
| rebound excursion | Displacement measurement | Primary return coordinate | Must capture maximum rebound and timing | Calibrated; synchronized | Peak excursion + timing |
| temperature | Thermocouple / appropriate sensor | Absorber/joint/ambient as justified | Only when temperature materially affects interpretation | Calibration + environment record | Raw temperature + correlation |

Instrumentation hard rule: a sensor class is not “adequate” merely because it exists. Adequacy requires demonstrated range, bandwidth, resolution, installation integrity, calibration, reference frame, synchronization and uncertainty.

# F. P0_FAILURE_MODE_REGISTER

| Failure mode | Detection | Stop condition | Post-test inspection |
| --- | --- | --- | --- |
| Rail binding | Motion discontinuity, asymmetric resistance, video | Any unstable / unintended motion | Inspect rails, guides, surfaces, witness marks |
| Unintended contact | Video + contact witness + unexpected force signature | Unexpected load-path change | Inspect contact surfaces and clearance |
| Lock failure | Lock-state channel + visual confirmation | Loss of restraint | Inspect latch and mating features |
| Premature lock transition | State timing inconsistent with intended sequence | Unsafe/uncontrolled transition | Inspect timing and mechanism surfaces |
| Failure to unlock | State + mechanical inspection | Mechanism traps motion / compromises test | Inspect lock and actuator path |
| Absorber bottom-out | Stroke limit + force spike | Bottom-out invalidates condition or exceeds equipment limit | Inspect absorber and stroke interface |
| Absorber instability | Force/displacement oscillation or visual instability | Unstable trajectory | Inspect absorber and attachment |
| Rebound overshoot | Peak excursion / velocity / video | Uncontrolled rebound | Inspect rebound mechanism / stops |
| Joint yielding | Permanent deformation / deformation channel | Unexpected loss of structural function | Measure permanent set; inspect joint/hardware |
| Bolt slip | Relative motion / witness marks / preload change | Load path changes unpredictably | Inspect torque/preload and interfaces |
| Excessive permanent deformation | Post-test dimensional check | Structural function compromised | Dimensional inspection |
| Structural instability | Video + sensor signature | Any unstable structural mode | Full post-test inspection |
| Fixture movement | Fixture reference channel / video | Fixture migration | Inspect fixture attachment and datum |
| Sensor saturation | Channel clipping / limit flag | Interpretation or safety compromised | Review channel, recalibrate/retest |
| Data synchronization loss | Reference pulse / timing cross-correlation | Causal timing cannot be trusted | Reject affected causal interpretation; recover/retest |

# G. TEST ARTICLE REPEATABILITY

Two tests are the SAME CONFIGURATION only when geometry, assembly torque/preload, absorber state, lock state, lubrication/surface condition, material/part identity, relevant temperature, instrumentation installation, boundary condition and initial condition are identical or explicitly documented as controlled equivalents.

| Test class | Purpose | Repeat policy |
| --- | --- | --- |
| SINGLE CHARACTERIZATION | First physical characterization / mechanism identification | One run can establish an initial signature when no statistical claim is made. |
| REPEATABILITY TEST | Quantify setup/article variability | Use repeated runs sufficient to characterize practical variability; do not choose a number merely to manufacture confidence. |
| A/B COMPARISON | Discriminate a single design variable | Replicate enough to separate design effect from observed setup variability; document sequence effects. |
| FAILURE TEST | Expose failure mechanism / boundary | Repeat only when scientifically useful and safe; preserve failed article evidence. |

# H. DATA INTEGRITY CONTRACT

| Field | Required | Rule |
| --- | --- | --- |
| TEST_ID | YES | Unique, immutable test identifier |
| ARTICLE_SERIAL | YES | Physical article traceability |
| CONFIGURATION_ID | YES | P0-A/B/C/D/E identity |
| DESIGN_REV | YES | Prototype revision identity |
| PROCEDURE_REV | YES | Released procedure revision |
| DATE / OPERATOR | YES | Execution trace |
| INSTRUMENT_ID | YES | Channel-to-hardware mapping |
| CALIBRATION_ID | YES | Calibration trace |
| RAW_DATA_HASH | YES | Cryptographic raw-data identity |
| VIDEO_HASH | Where applicable | Video evidence identity |
| ENVIRONMENT | YES | Temperature and other relevant conditions |
| SETUP_RECORD | YES | Fixture, assembly, sensor placement |
| RESULT | YES | Reduction result with status |
| UNCERTAINTY | YES | Measurement uncertainty / processing basis |
| DEVIATIONS | YES | All deviations from released procedure |
| FAILURE_LOG | YES | Failure/stop events |

Handwritten or untraceable results are not authoritative evidence.

# I. P0_R4.1_TRACEABILITY

| Trace class | Definition | Use |
| --- | --- | --- |
| R4.1-DIRECT | P0 implementation directly carries the R4.1 feature/intent | Highest correlation category |
| R4.1-SIMPLIFIED | P0 simplifies geometry or integration while preserving the mechanism being investigated | Valid for mechanism learning; correlation impact must be recorded |
| P0-EXPERIMENTAL | Feature intentionally added to test a hypothesis | Evidence applies to the experimental feature, not automatically to R4.1 |
| P0-TEST-FIXTURE | Feature exists only for fixture/instrumentation/test execution | Excluded from mechanism claims unless explicitly relevant |

| Required trace fields | Mandatory content |
| --- | --- |
| R4.1 SHA256 | fbe6b17cdbf7282a2e47963e567e12eeceb1352a36e719e7d1c55cc5f712a0a68 |
| R4.1 target | Feature / function being traced |
| P0 feature | Actual implemented feature |
| Deviation status | Direct / simplified / experimental / fixture |
| Reason | Why deviation exists |
| Expected physical effect | What the deviation may change |
| Correlation impact | None / local / significant / unknown |

# J. TEST → INPUT CLOSURE MAP

| Physical input / property | P0 source | Status | Legitimate boundary |
| --- | --- | --- | --- |
| Mass | P0-A L1 measurement | GENERATED_BY_P0 | Directly measurable on physical article |
| CG | P0-A L1 measurement | GENERATED_BY_P0 | Only if measured by a valid procedure |
| Inertia | P0-A dedicated measurement if needed | GENERATED_BY_P0 (conditional) | Do not infer from CAD alone as test evidence |
| Density | Material characterization / measured geometry | GENERATED_BY_P0 (where legitimately measurable) | Only for materials/properties actually characterized |
| Friction | L2/L3/L4 contact characterization | GENERATED_BY_P0 | Requires controlled contact condition and method |
| Contact stiffness | L3/L4 | GENERATED_BY_P0 | Only where contact event is physically characterized |
| Damping | L3/L4 response identification | GENERATED_BY_P0 | Identified from measured response; not assumed |
| Restitution | L3/L4 where valid impact exists | GENERATED_BY_P0 (conditional) | Only if a real impact/contact event is measured |
| Absorber F-x | P0-B L3 | GENERATED_BY_P0 | Direct test characteristic |
| Absorber F-v | P0-B L4 | GENERATED_BY_P0 | Requires dynamic measurement |
| Joint compliance | P0-D / L3-L4 | GENERATED_BY_P0 | Measured deformation/load relationship |
| Bolt stiffness | L3 characterization where valid | GENERATED_BY_P0 (conditional) | Requires dedicated valid method |
| Vehicle-specific pulse | Not generated by bench P0 | NOT_GENERATED_BY_P0 | Requires separate validated vehicle/sled input |
| 14th input from source master list | Source list not present in current controlled material | NOT_IDENTIFIABLE / BLOCKER | Must be recovered from the authoritative 14-input definition; do not invent |

# K. P0_DESIGN_DECISION_MATRIX

| Observed result | Interpretation | Design action |
| --- | --- | --- |
| PASS / criterion met | A pre-defined criterion is demonstrated | Retain configuration |
| FAIL | Defined criterion or safe operation not achieved | Identify mechanism → modify relevant component → retest |
| AMBIGUOUS | Experiment cannot isolate cause or timing | Redesign experiment; no causal inference |
| NO DIFFERENCE | Effect not distinguishable from variability/uncertainty | Check sensitivity, calibration, repeatability and sample adequacy; do not declare equivalence |
| INCONSISTENT | Repeated configuration behaves materially differently | Investigate assembly, setup, fixture, material or intrinsic nonlinearity |
| SENSOR-INCONCLUSIVE | Measurement cannot discriminate hypothesis | Fix instrumentation; rerun |
| SAFETY-FAIL | Physical behavior violates safe test envelope | Stop; inspect; root-cause before further testing |

# L. STOP CRITERIA

Unexpected structural failure

Unstable motion

Fixture migration

Loss of restraint

Sensor saturation that compromises safety or interpretation

Uncontrolled rebound

Unexpected lock behavior

Any condition exceeding defined equipment/test limits

Stop sequence: SAFE STATE → PRESERVE DATA → INSPECT → FAILURE LOG → ROOT-CAUSE REVIEW → MODIFY ONLY AS JUSTIFIED → RETEST.

# M. PRE-FABRICATION CRITICAL REVIEW

| Domain | Status | Evidence / next action |
| --- | --- | --- |
| Function | TEST_REQUIRED | L2 functional sequence |
| Kinematics | TEST_REQUIRED / CAD-CONTROLLED | Native CAD geometry + L2 evidence |
| Load path | TEST_REQUIRED | S1/S2 and physical load-path measurement |
| Joints | TEST_REQUIRED | Preload, compliance and deformation evidence |
| Contacts | OPEN until CAD/inspection | Controlled clearance + witness inspection |
| Absorber | DESIGN_HYPOTHESIS | P0-B characterization |
| Lock | DESIGN_HYPOTHESIS | P0-C transition test |
| Rebound | DESIGN_HYPOTHESIS | P0-D dynamic characterization |
| Stroke | OPEN until source geometry verified | Critical stroke dimensions from CAD |
| Clearance | OPEN until source geometry verified | Critical clearances from CAD + L1 |
| Modularity | CLOSED at architecture level | Experimental modules defined |
| Instrumentation | OPEN | Actual sensor models/ranges/bandwidth/calibration must be released |
| Fixture interface | OPEN | Critical interface geometry and equipment limits required |
| Manufacturability | OPEN / CAD-dependent | Released fabrication definition required |
| Inspection | OPEN | L1 checklist and measurement methods required |
| Safety | OPEN | Actual equipment/test limits must be set |
| Repeatability | CLOSED at rule level | Article/setup identity definition established |
| Test discrimination | CLOSED | A/B variable isolation and P0-E non-causality established |

# N. UNRESOLVED BLOCKERS

| ID | Blocker | Why it matters |
| --- | --- | --- |
| B-01 | Authoritative native P0 CAD / drawing package | Exact critical geometry cannot be invented. Import the controlled source before cutting structural/kinematic features. |
| B-02 | Authoritative 14-input master list | The supplied material names 13 identifiable inputs; the 14th must come from the source definition, not inference. |
| B-03 | Released instrumentation BOM / channel spec | Sensor adequacy cannot be certified without actual range, bandwidth, resolution, calibration and installation definition. |
| B-04 | Fixture interface and equipment limits | Stop criteria must be tied to the real fixture/DAQ/actuator/restraint capability. |
| B-05 | P0 deviation register instantiated against actual source features | The deviation framework is defined; actual feature-by-feature entries require the controlled P0 source. |

# O. EXACT MANUFACTURING HANDOFF STATUS

| Item | Status | Release rule |
| --- | --- | --- |
| P0 exploratory development state | AUTHORIZED | Proceed as development workstream |
| P0-V0 fabrication definition | CONDITIONAL | Release only defined features with critical geometry and interfaces controlled |
| Production design release | NOT APPLICABLE | P0 is not production |
| Exact R4.1 reproduction | NOT CLAIMED | Use only where traceability demonstrates equivalence |
| P0-E integrated test | CONDITIONAL | Only after relevant subsystem evidence exists |
| Sled / higher energy | CONDITIONAL | Requires evidence-based separate gate |

# P. EXACT FIRST PHYSICAL ARTICLE: P0-V0

P0-V0 is the first buildable article that maximizes learning while preserving the intended mechanism under test. Its physical definition is:

Primary structural/base interface with controlled fixture attachment.

S1/S2 translation mechanism in the source-defined geometry.

Seatback structure and rotation-control interface.

Replaceable absorber interface and cartridge provision.

Replaceable lock interface and state-sensing provision.

Replaceable/adjustable rebound-control interface.

Controlled joint/preload interfaces.

Defined stroke/stops and safe end conditions.

Instrumentation brackets referenced to fixed datums.

Inspection access and post-test dimensional reference points.

No dimension is invented here. Critical dimensions must be instantiated from the native P0 CAD / controlled source. Locally controlled dimensions are limited to test hardware that does not change kinematics, load path, contact, lock engagement, or stroke.

# Q. FIRST TEST CAMPAIGN

| Sequence | Article | Test | Objective | Gate to next step |
| --- | --- | --- | --- | --- |
| 1 | P0-V0 | L1 inspection | Verify build state, interfaces, clearances, preload, sensors and fixture | No critical discrepancy |
| 2 | P0-V0 | L2 functional motion | Prove physical movement, lock, rotation, rebound, reset and absence of obvious binding | Stable repeatable function |
| 3 | P0-A | L3 baseline characterization | Establish physical baseline signatures | Valid synchronized data |
| 4 | P0-B | L3/L4 absorber A/B | Discriminate absorber behavior | Effect measurable or experiment declared inconclusive |
| 5 | P0-C | L2/L4 lock A/B | Discriminate lock transition behavior | Effect measurable or redesign experiment |
| 6 | P0-D | L2/L4 rebound A/B | Discriminate rebound-control behavior | Effect measurable or redesign experiment |
| 7 | P0-E | L5 integrated | Characterize system interaction; no component causality | Evidence sufficient to justify next design iteration |

# R. PROTOTYPE ITERATION CONTROL

| Revision | Trigger | Required record | Next action |
| --- | --- | --- | --- |
| P0-V0 | First article | Baseline configuration, CAD revision, traceability, setup | Test |
| P0-V1 | Physical failure or measurement-backed improvement opportunity | Change list + rationale + expected effect | Targeted retest |
| P0-V2+ | Subsequent evidence | Same change-control discipline | Continue until mechanism converges or gate decision changes |

A design revision must never be created merely to erase an inconvenient result. Failed articles and failed tests remain part of the evidence chain.

# S. FINAL JON ENGINEERING DECISION

P0 DOE / TEST DISCRIMINATION = PASS. The test program is capable of producing actionable evidence because each component A/B experiment has one primary design variable, defined control variables, measurable outputs, and a downstream design decision. P0-E is correctly limited to integrated system evidence.

P0 EXACT FABRICATION RELEASE = BLOCKED. Not because the concept has failed, but because exact article-defining source data are not present in the controlled material available here. The correct action is to import the native P0 source, instantiate critical dimensions/interfaces, release instrumentation and fixture limits, then fabricate P0-V0.

Next operating state: FABRICATE → INSPECT → TEST → MEASURE → ANALYZE → MODIFY → RETEST.

# T. DOCUMENT CONTROL

| Field | Value |
| --- | --- |
| Document | JON_FIRST_REAL_PHYSICAL_TEST_PROTOTYPE_MASTER |
| Role | Development / prototype iteration master |
| R4.1 revision | IMMUTABLE |
| R4.1 SHA256 | fbe6b17cdbf7282a2e47963e567e12eeceb1352a36e719e7d1c55cc5f712a0a68 |
| P0 hardware status | NOT YET BUILT |
| FE/MBD | BLOCKED |
| P0 exploratory fabrication | AUTHORIZED IN PRINCIPLE |
| Exact fabrication | BLOCKED pending native source control |
| R4.2 | NOT CREATED |